Problem Set 3 ENV 535 (Due April 22)

- A field test was performed to estimate contaminant loading to groundwater due to land application of liquid effluent. Water is applied on a plot at an average darcy velocity of 0.0023 m/d with ln estimation error sln = 0.2. The average volumetric water content is estimated from core sample to be 0.15 with estimation error s = 0.01. Soil bulk density is ~1600 kg/m3, soil organic carbon content is 0.005, and the koc for the chemical of concern is 0.025 m3/kg. Compute the soil adsorption coefficient from this data and assume pooled ln estimation error of sln = 0.4. The reactive chemical is assumed to undergo biodecay that will be approximated by first-order decay. A rough guess for the aqueous first order decay coefficient is 0.001 1/d with sln = 2. No decay is assumed to occur in the adsorbed phase. Tracer and reactive chemicals are added simultaneously at constant concentrations of 10 and 100 g/m3, respectively, for a 30 day period, after which clean water is applied. Dispersivity is assumed to be the same for both chemicals with a prior estimate of 2.5% of the max travel distance with uncertainty sln = 2. Flux concentrations are measured with solution samplers at the water table at 2 m depth over a 500 day period beginning with the initial application and resident concentrations are measured at various depths at 150 and 300 days. (see data in CXTFIT worksheet).

  - Estimate the darcy velocity, water content, dispersivity, and aqueous decay coefficient for the reactive chemical, their estimation errors and correlation matrix from the inverse solution.

  - Estimate 95% confidence limits ( = 0.05) for the steady state mass flux to the water table from a 1000 m2 application area (in g/d) of the reactive chemical for the test conditions with a constant loading concentration of 100 g/m3.

  - Estimate  95% confidence limits ( = 0.05) for the steady state mass flux to the water table from a 1000 m2 application area (in g/d) of the reactive chemical applied at a constant concentration of 50 g/m3 with the darcy velocity 5x higher and water content 1.2x higher than the test conditions.

- A TCE DNAPL release from a former manufacturing plan occurred around the 1960s and has resulted in a large aqueous plume. You need to come up with a remediation plan that can get regulatory approval at the least cost. Regulators want a plan that will shrink the plume such that by 2040 TCE concentrations in all wells greater than 1000 m from the source will be less than 10 g/L. If this condition is not met, a pump and treat system will have to be installed. Monitoring well data is available for calibration in addition to prior information from various sources pertaining to various aquifer and plume parameters, as summarized in a DNAPLplume worksheet.

  - Perform model calibration using the available data, estimated parameter covariances. Estimate the probability of meeting the regulatory criteria with only natural attenuation at the site.

  - Monitoring and reporting will cost $100K per year until regulatory criteria are met. Active remediation options under consideration include partial source removal and enhanced bioremediation with emulsified vegetable oil injection. Thermal source treatment is capable of removing 95% of the remaining DNAPL mass in the source at a cost of $500K. Vegetable oil injection in one or more injection well galleries can be performed for $3K per m of injection gallery width plus $1K per kg of injected oil. A pilot test indicates that oil dissolution can be described by the NAPL source model with Jcal=0.02 kg/d with tcal=injection date, and =1. One kg of veg oil can induce the biodecay of about 10 kg of TCE. Assuming fast reaction kinetics, the concentration of TCE following ED injection can be estimated by superposition as C’TCE = CTCE – REDFEDCED/RTCE where CTCE is the TCE concentration computed without the ED reaction, CED is the ED concentration computed without reaction, FED=10 is the reaction stoichiometry factor, and RED and RTCE are respective retardation factors. Assume retardation for ED to be 2. ED injection can be performed in one or more galleries at one or more times. If ED injection is performed upgradient of the source, the DNAPL source mass transfer rate may be assumed to increase by a factor of 3 (with Sln=0.3). Future costs may be discounted at a rate of 2% to compare expected total costs in net present value (NPV). In addition to the known costs for the planned active remediation measures, the possible cost of pump and treat (assume $5M in NPV) should be weighted by the probability that your plan may fail and added to the total cost.