Instructions for Performing Inverse Solutions

and First-Order Error Analyses with CXTFIT Excel 2007

Jack Parker April 2009

- Open the CXTFIT file.

- Click “Options” in the start up warning bar and check “Enable the content”

- Click the Office button, select “Excel options.” Verify that “Show developer tab in ribbon” is checked. Confirm Solver Add-In is active (if not activate or install as required)

- Set up the worksheet to compute the objective function.

  - Set up block in sheet for parameters that are to be estimated with the prior parameter estimate (or ln of estimate if log-transform is used), std deviation of prior estimate (or std dev of ln parameter), weighted parameter deviation (difference between current estimate and prior estimate/prior SD), and the current parameter estimate (or its ln). Initial values in the current estimate fields should be set to the prior estimate. Leave a block unused to the right of the current estimate column of 3+Np cells where Np is the number of parameters to be estimated.

  - Set up a block with input parameter required for the direct solution calculations. Parameters that are to be estimated or that are functions of estimated parameters should be computed from values in the current parameter estimate fields (see above). If estimated parameters are log-transformed, remember to exponentiate them to return to the real units!

  - Set up a block with observations that will be used for the inverse solutions in a column and and adjacent column with predicted valued computed from the relevant direct solution function (e.g., CDE or MIM). The function calls should reference relevant fixed parameters and current values of estimated parameters. Add additional columns with the standard deviation of the observed data, the deviation between observed and predicted values, and the weighted deviation (=unweighted deviation/SD). If predicted are to be log-transformed, zero values for observation and predictions must be avoided. This is best accomplished by defining a “minimum detection level” (MDL) below much the value may be regarded as insignificant. Replace any zero observations with the MDL and modify the predicted field to be the greater of the function call or the MDL. If results are log-transformed, note that the SD is the ln SD, and the deviation values are ln deviations (=ln observed – ln predicted).

  - If forward predictions for conditions different from the calibration data set are to be made, additional rows may be added below the calibration data, set up in the same manner but with “observations” set to the MDL and with “predictions” computed using parameters, boundary conditions, etc. pertinent to the forward problem. A large fictitious SD should be specified (e.g., 1E10) to eliminate the forward problem from the objective function.

  - If prediction error is to be computed, add additional column to the right of the observed and predicted results for the parameter propagation variance, regression variance, total prediction standard deviation, lower confidence limit and upper confidence limit. The parameter propagation variance will be computed using the CXTFIT first order error propagation module. The other fields may be computed as described in the technical documentation.

  - Finally, set up a field to compute the objective function by adding the weighted deviations for observations and the penalty function for parameters.

- Open the “Add-Ins” menu, select “CXTFIT” and “Solve” then enter cell location with objective function, cell range for parameters being estimated, and cell range for model predictions (including forward predictions if applicable).

- Still in the Solve module, select “Options”

  - If you wish to specify min-max constraints for parameters, check the first box and input min and max value in the two columns immediately right of the “estimate” field. Otherwise, disregard.

  - Check second box and indicate the column offset between the parameter estimate column and the prior SD column. Right is positive. (e.g., if parameter estimates are in column J and the Prior SD values are in column H, the offset is -2)

  - If you want to manually specify perturbation factors for the sensitivity matrix, check the third box and indicate the offset to the column with the factors. The default perturbation is 0.01 of the estimate. The default may give poor results if the parameter is a large value with a small SD (e.g., if release date for a plume is estimated with a prior of 1970 and a prior SD of 5 years, 1/1970 = 0.0025<<0.01).

  - To perform error analyses, check to fourth box and input the offset between the observation SD and the predictions (e.g., if prediction are in column G and SDs are in column H, the offset is +1).

  - If it is desired to give the penalty function a relative weight, check the last box and indicate the value or cell location for the weight.

- Press the “Solve” button to execute the inverse solution. If the solution converges, accept the solution. Otherwise revert to the initial values, modify settings and try again. After the inverse solution is completed the Solve function will compute parameter standard errors and the correlation matrix.

- Select the “Propagate parameter uncertainty” module from the CXTFIT Add-In

  - Confirm parameter and prediction cell ranges

  - Specify the offset from the prediction column to the column with parameter propagation variance column.

  - To output the sensitivity (Jacobian) matrix) check the first box and indicate the offset. (Note the Jacobian will take up a number of columns equal to the number of estimated parameters)

  - See 6c above.

  - Press “Calculate” to compute and output the parameter propagation variance

- To compute the total prediction error add the parameter std deviation (=sqrt(variance)) to the regression variance, which can be computed directly in Excel.

- Confidence limits can then be computed from the predicted values and the total error estimate for a given probability level for the tails; i.e., for 95% CL, alpha = 0.05. Note that the t-statistic for two-tailed confidence limits may be computed in Excel as TINV(alpha, N) where alpha is the probability level (e.g., 0.05 for 95% CL) and N is the degrees of freedom (number of observation used in inversion minus the number of fitted parameters). For on-tailed confidence limits the t-statistic is TINV(2*alpha, N)